Single Quantum Radio Interferometry

Ilia Platone

E-mail: info@iliaplatone.com

Abstract

Radio interferometers primarily adopt amplitude interferometry to obtain science correlations and data. Amplitude interferometry streams or data are required to contain both real and imaginary parts of the continuum observed, thus IQ sampling records are used for this purpose and large amount of data are required to store them.

Intensity interferometry uses 2nd or greater order of the spatial coherence of flux intensities, and it discards the phase of the current observation, correlating only quantum correlations quantities. For this reason this observing technique can be taken into account to build low-budget interferometers and the even-like nature of streams used obtained permit single-bit streams to be stored and to minimize observation size and greater portability or further processing.

Here is proposed a correlation technique which implements an high pass amplifier and a passively-tuned antenna. The amplifier Is tuned to its bandwidth limit. This minimum energy quantity is tried to be amplified as electronic pulses and then sent to a cross-correlator for each baseline and then permits further processing like radio imaging and spectrum analysis, or other processing techniques.